Annex 2

TECHNICAL SPECIFICATION

For the order of supply titled

“Delivery of Complete Laser Technology Set III.“

The subject of the present specification is supply of a complete laser technology set comprising a disc diode laser source, a cutting head, an optical fiber and a cooler. The design of the complete set should be conceived for a cutting machine building equipped with a solid substance (disc diode) laser technology permitting cutting of various metal materials including at least black steel, stainless steel, aluminium, brass, copper and galvanized metal.

The table below defines the required technical parameters of the individual components of the supply. The parameters are defined either as minimum or maximum, or a range or an exact value or feature. The parameters are defined for one line.

The bidder shall enter in the empty fields:

- In the case of enumerable parameters: the respective numerical value (corresponding to the required minimum, maximum or accurately defined value)

- In the case of non-enumerable parameters: YES/NO depending on whether the offered device meets/does not meet the requirement.

If the bid does not meet the parameter requirements (i.e. in the case of enumerable parameters the required value is not met and in the case of non-enumerable parameters the answer is NO) then the bidder shall be excluded from the tender.

| Parameter | Minimum requirement | Offered parameter value, or enter “yes” or “no” |
| --- | --- | --- |
| Set of the fiber laser technology |  |  |
| Laser power | min. 3kW |  |
| diode pumped disk laser | Yes |  |
| Laser source wavelength | min. 1030 nm |  |
| The length of the optical fiber | min. 20 m |  |
| The optical fiber diameter | 100/200 micron |  |
| Digital I/O interface | Yes |  |
| On line piercing monitoring | Yes |  |
| The ability to service the laser source directly at the installation location on the machine | Yes |  |
| Online servicing with team viewer | Yes |  |
| Data reach via a bus or bluetooth | Yes |  |
| Active laser power control for high power stability of +/- 1% including measurement data collection and reverse lookup | Yes |  |
| Energy saving module | Yes |  |
| Internal cooling circuit with water-water, heat exchanger and status indication | Yes |  |
| Optional direct communication between the laser source and the cooler | Yes |  |
| Relative humidity max 100% at 27°C | Yes |  |
| Ambient operation temperature 10 – 50° C | Yes |  |
| Collimation of focal lenght 100 mm | Yes |  |
| Protective glass in collimation | Yes |  |
| Optional industrial communication through Ethercat | Yes |  |
| Optional industrial communication through Analog | Yes |  |
| Cutting Head |  |  |
| Maximum permissible power of laser cutting head | min. 3 kW |  |
| Motorized cutting head focus | Yes |  |
| Optional control of cutting head laser beam width | Yes |  |
| Optional piercing detection | Yes |  |
| Cutting head with nozzle pressure monitoring | Yes |  |
| Cutting head with capacitive sensing temperature monitoring, stable, flicker-free | Yes |  |
| Internal optics protection | Yes |  |
| Sensor for height adjustment from material including regulator | Yes |  |
| Protection of the internal optics of the cutting head through the lower glass | Yes |  |
| Protection of the internal optics of the cutting head through the upper glass | Yes |  |
| Capacitive sensor of cutting head | Yes |  |
| Evaluation of the capacity sensor of the cutting head in the PLC machine | Yes |  |
| Optical fiber connection to cutting head | Direct |  |
| Cutting head weight | max. 8 kg |  |
| One head for 2D and 3D cutting | Yes |  |
| Optional industrial communication through Ethercat | Yes |  |
| Optional industrial communication through Analog | Yes |  |
| Cooler for 3 kW laser |  |  |
| Dual-circuit laser technology cooler | Yes |  |
| Accessories |  |  |
| All accessories (cables, hoses) for connecting the laser source to the cooler included | Yes |  |
| All accessories (cables, hoses) for connecting the cutting laser head to the cooler included | Yes |  |
| All accessories (cables, hoses) for connecting the cutting laser head to the laser source included | Yes |  |

| Bid items | Identification/manufacturer |
| --- | --- |
| Disc diode laser source |  |
| Cutting head including optical fiber |  |
| Cooler |  |

Bidder´s statement

The bidder ******* hereby claims that the offered components of the laser set are mutually compatible and do not require any further mutual SW programming.

In 	, on

…………………………………………………………………………………………

Signature of authorized representative of bidder